Experiment 08:

Name of the Experiment: Voltage Divider Biasing of Enhancement Type MOSFET

Theory:

MOSFETs

A Metal–Oxide–Semiconductor Field Effect Transistor (MOSFET) is a type of Field Effect Transistor (FET) that operates with voltage control at the gate terminal. Unlike BJTs, which are current-controlled devices, MOSFETs use an electric field induced by the gate-to-source voltage (VGS) to control the current flowing from drain to source (ID). The MOSFET has three terminals: Gate (G), Drain (D), and Source (S). In an n-channel Enhancement-type MOSFET (E-MOSFET), no drain current flows when V. Conduction begins only when. Above this threshold, the drain current follows the quadratic relationship:

ID=kVGS-Vth2

Here,

ID is the drain current.

The VGS is the Gate to Source Voltage.

The Vth is the threshold voltage at which the MOSFET turns on

k is the transconductance parameter which solely depends on the type MOSFET being used.

Biasing Techniques

To set a proper operating point (Q-point) for MOSFET circuits, several biasing techniques can be used.

The Self Biased Arrangement (Feedback biasing arrangement): This circuit, designed in figure 7.1, shows how the self-biased circuit works. The idea is to connect a very high resistance between the drain and gate which can draw huge of voltage and supply it to the gate terminal. Since the current IG=0 A, the current in flowing through RD remains the same.

Fig 7.1:

Feedback Biasing Arrangement

Fig 7.2:

DC equivalent of Fig 7.1

Fig 7.2

Voltage Divider Biasing Circuit: The circuit given in Figure 7.3, shows how the voltage divider circuit works. What this circuit does is that since IG=0 A, it takes the voltage VDD and divides it amongst R1 and R2, thus, using Voltage Divider Rule, the gate voltage VG applied will be equal to the voltage across R2.

VG=R2R1+R2VDD

Using KVL in the given loop in Figure 7.3, we can get the following equation:

VGS=VG-IDRS

Recall that drain current ID flowing through RD and RS are the same.

Fig 7.3:

Voltage Divider Biasing Arrangement

Transfer Curve

The Transfer Curve for an n-type MOSFET can be found using Shockley’s equation, the graph in Figure 7.4 shows a sudden exponential increase of drain current when the gate-to-source voltage applied is greater than the threshold voltage VGS>Vth.

Fig 7.4:

Transfer Curve for n-channel Enhancement type MOSFET

This curve is obtained after knowing the parameter k, which can be found using the equation:

k=ID(on)VGSon-Vth2
The drain current and gate voltages needed to calculate k, are taken from the data sheet of the MOSFET being used.

Q-Point

The Q-point (short for Quiescent Point) is the operating point of a transistor when no input signal is applied. For this lab, it represents the steady-state values of drain current ID and gate-to-source voltage (VGS) that are set by the biasing network.

Which means, after building the biasing circuit in the lab, your measurements will be your Q-point values. However, it is also important to find the theoretical Q-point of a MOSFET from the transfer curve by applying load line analysis as shown in Figure 7.5.

VGRS

Fig 7.5:

Load Line Analysis on the Transfer Curve for n-channel Enhancement type MOSFET

Equipment and Component:

| Serial no. | Component Details | Specifications | Quantity |
| --- | --- | --- | --- |
| 1 | MOSFET | IRF540 | 1 piece |
| 2 | Resistor | 1kΩ, 470Ω, 5kΩ | 1 piece each |
| 3 | Potentiometer (POT) | 10kΩ | 1 piece |
| 4 | Trainer Board |  | 1 unit |
| 5 | DC Power Supply |  | 1 unit |
| 6 | Digital Multimeter |  | 1 unit |
| 7 | Chords and Wires |  | 1 unit |

Experimental Setup 1:

Procedure:

- Build the circuit as per the setup.

- Set VGS to zero and VDS = 15V measure ID.

- Increase the gate voltage VGS gradually and record the readings.

- Take reading until ID suddenly increases and goes to 15 mA.

- Note the value of VGS, and ID (approximately 15 mA) and write it down in Table 7.1.

- Calculate the value of k.

Table 7.1: Calculation to find k

| VGS(on) | IDon | Vth | k=ID(on)VGSon-Vth2 |
| --- | --- | --- | --- |
|  | 10 mA | 3 V |  |

Experimental Setup 2:

Procedure:

- Connect Resistors R1 = 5kΩ, RD=1kΩ, RS=470Ω and set up the circuit as shown.

- For R2, connect a 10kΩ potentiometer and set it to minimum, and set V1=18V.

- Measure VG by putting one probe of the DMM to Gate terminal and the other probe to ground.

- Gradually Increase R2 until VG = 8V.

- Note the value of VGS, and ID and write it down in the table.

Table 7.2: Finding the Q-points

| Measured |  | Calculated |  |
| --- | --- | --- | --- |
| VGSq | IDq | VGSq | IDq |

Report:

- Draw the transfer curve based on the data that you have calculated in Table 1.

- Find the Q-points from your graph and check whether your Measured data matches with the data you found.

- Does the Shockley equation depend on the type of biasing circuit used or the type of MOSFET being used? Explain.